# Extended Libraries

Schleipen, Miriam, Siemens AG, Karlsruhe, miriam.schleipen@siemens.com

## How to use AutomationML RoleClass Libraries

AutomationML includes different library types including the RoleClassLibraries. But how to approach all these extended libraries existing in the AutomationML “cosmos” and make use of them? Let us focus on the usage of RoleClass libraries as one representative of the AML libraries.

RoleClasses in AutomationML leverage the unified definition of semantics – key enabler for interoperability – and provide a means to re-use them within AutomationML models. The basic concept of RoleClasses provides the possibility to define a common meaning of automation entities and concepts.

Each RoleClass shall have a unique name within a RoleClass library. This results in a hierarchy path which can be used for referencing this RoleClass.

Each RoleClass may have attributes and interfaces defining details about the underlying concept. They are organized in hierarchical structures called RoleClass libraries and contain an attribute RefBaseClassPath which is used to define their semantic parent. The semantic parent is not necessarily the parent element in the RoleClass library, but can also reference a RoleClass within another RoleClassLibrary.

RoleClasses shall always have a defined parent. The minimum requirement given by AutomationML is to derive your role from the AutomationMLBaseRole which is included in the AutomationMLBaseRoleClassLib and forms the semantic base of all RoleClasses. Even if individual user-defined RoleClasses are defined by one partner, the other partner can try to interprete this unknown RoleClass by searching for some known RoleClass within the semantic parents from which this RoleClass is directly or indirectly derived. As all RoleClasses have the AutomationMLBaseRoleClass as semantic parent, this is the general fallback if all derived RoleClasses cannot be understood by both partners who exchange information in an AutomationML model.

There are different stages of unified semantics which can be chosen as required:

- User-defined RoleClass libraries in one single model (see 1.1.1

- User-defined RoleClass libraries in multiple models (see 1.1.2)

- User-defined RoleClass libraries in a defined context (see 1.1.3)

- AutomationMLBaseRoleClassLib (see 1.1.4)

- Normative AutomationML IEC RoleClass libraries (see 1.1.5)

- Informative AutomationML IEC RoleClass libraries (see 1.1.6)

- AutomationML e.V. RoleClass libraries (see 1.1.7)

General remark: Please note that the RoleClass libraries present in the current edition of [IEC62714-2 2015] are still compliant to CAEX version 2. The RoleClass libraries compliant to CAEX version 3 will be included in the upcoming second edition of [IEC62714-2 2015], but are used in the section 1.3 and thus included in the [LINKAMLExample].

### User-defined RoleClass libraries in one model

You can define proprietary RoleClasses within RoleClass libraries related to your specific model and re-use them in this model. Good practice is to add a description to all RoleClasses to be sure that the intended semantic is understandable. These RoleClass libraries are called user-defined in the context of AutomationML. An example might be a special LiftingTable RoleClass of a lifting table type which is included one or multiple times in your production line. The LiftingTable RoleClass is derived from AutomationMLBaseRole.

### User-defined RoleClass libraries in multiple models

You can bring them to the next level and re-use them in other models. For this, you simply include your proprietary RoleClass library with all RoleClasses in the new models.

An example might be the LiftingTable RoleClass of a lifting table type which you use in more than one production line and thus appears in multiple AutomationML models.

### User-defined RoleClass libraries in a defined context

You can exchange these user-defined RoleClass libraries with others, e.g. within your company, to share the common understanding (semantic) of the defined RoleClasses. Good practice is to collect or define RoleClass libraries in a defined context, e.g. one department or company, and use the AutomationML mechanism for versioning (described in IEC62714-1 [IEC62714-1 2018], clause 5.3) to avoid conflicts with emerging RoleClass libraries over the time.

An example might be a LiftingTable RoleClass of a lifting table you use in all your factories.

### AutomationMLBaseRoleClassLib

You can use the RoleClasses defined in the AutomationMLBaseRoleClassLibrary within the IEC 62714-1 [IEC62714-1 2018] (see Fig. 1.2). This results in a minimum of interoperability as you can already distinguish between product, process, resource, and other basic entities and concepts.

An example might be a lifting table RoleClass which is derived from the Resource RoleClass of the AutomationMLBaseRoleClassLib to depict that this model element represents a production resource instead of a product or process.

Fig. 1.1: AutomationMLBaseRoleClassLib

### Normative AutomationML IEC RoleClass libraries

You can use the normative RoleClass libraries defined in the IEC 62714-2 [IEC62714-2 2015], IEC62714-3 [IEC62714-3 2017], and IEC62714-4 [IEC62714-4 2020]. Normative means in this case that their meaning is defined in a normative clause of the standard document and is a must if you want to be compliant with the IEC standard. The normative RoleClass libraries of IEC62714-2 [IEC62714-2 2015] (see Fig. 1.2) include libraries for discrete manufacturing industry (AutomationMLDMIRoleClassLib), continuous manufacturing industry (AutomationMLCMIRoleClassLib), batch manufacturing industry (AutomationMLBMIRoleClassLib), and control systems (AutomationMLCSRoleClassLib).

Fig. 1.2: AutomationML normative standard libraries

IEC62714-3 [IEC62714-3 2017] and IEC 62714-4 [IEC62714-4 2020] include normative RoleClass libraries necessary for representing geometry and kinematics or logic and behavior in AutomationML models.

An example might be a lifting table RoleClass which is derived from the Resource RoleClass of the AutomationMLBaseRoleClassLib to depict that this model element represents a transport equipment. If you want to analyze your material flow you can list all transport equipment of your model in this way.

### Informative AutomationML IEC RoleClass libraries

You can use the informative RoleClassLibrary defined in IEC 62714-2 [IEC62714-2 2015] (see Fig. 1.3) named ExtendedRoleClassLib. It includes a collection of additional concepts and entities with common semantics, but is not mandatory to be conform to IEC62714-2.

An example might be a lifting table RoleClass which is derived from the RoleClass LiftingTable of the AutomationMLExtendedRoleClassLib to depict that this model element represents a lifting table. If you want to analyze all you lifting tables you can directly search for all InternalElements referencing this RoleClass LiftingTable.

Fig. 1.3: AutomationML informative standard library

### AutomationML e.V. RoleClass libraries

You can use the RoleClassLibaries provided by the AutomationML e.V. which are related to modelling concepts described in Application Recommendations (AR), Best Practice Recommendations (BPR) or whitepapers [AML1].

An example might be a lifting table RoleClass which is derived from a specific LiftingEquipment included in the MaterialHandlingResourceClassLib (see Fig. 1.4) of the AR Modelling of Material Handling in AutomationML [AML4].

Fig. 1.4: MaterialHandlingResourceClassLib

## How to work with AutomationML RoleClass Libraries

Good practice to work with AutomationML RoleClass libraries is to start by checking if there are already existing libraries for your intended semantics.

Starting point are the standardized normative and informative libraries in the IEC series of AutomationML. The corresponding description can be found in the standard documents [IEC62714-1 2018], [IEC62714-2 2015], [IEC62714-3 2017], [IEC62714-4 2020].

They can be downloaded via the homepage of the AutomationML e.V. [AML2] or imported via the menu entry “File”“Import””from AutomationML” as depicted in Fig. 1.5 and Fig. 1.6 directly in the AutomationML editor.

If you download the libraries from the homepage, you can import the files via the menu entry “File”“Import””from Files”. This function of the AutomationML editor can be used as well to exchange user-specific libraries.

All this functionality provided by the AutomationML editor includes in the background the corresponding XML snippets into your XML file. You can also do this XML editing directly within the XML text (manually) without this tool support.

Fig. 1.5: Import Libraries within AutomationML editor

In case there are conflicting versions proposals for resolving them are presented to the user of the AutomationML editor to avoid inconsistencies. In case of manual editing the XML, the user must care about this.

If you cannot find a RoleClass with your intended semantic, check the information RoleClass Library in [IEC62714-2 2015] which is also included within the standard library package [AML2].

In case you couldn’t a matching RoleClass, check the RoleClass libraries defined by AutomationML e.V. for all ARs and BPRs described in the documents linked here [AML1] which can be downloaded in the corresponding libraries package [AML3].

Given the case that no RoleClass for your intended use case exists in these RoleClass libraries, you can create your own user-defined one.

Fig. 1.6: AutomationML editor Import dialog

## How to model a “plant” hierarchy with the ResourceStructure semantic

The base for an exchange of engineering information is to identify the elements included in a model. The identification can for sure be done via IDs of the AML elements. More often, the tools possess own identification mechanism and are only able to maintain the AutomationML ID during import and export to avoid inconsistencies. Normally you put the concrete model elements (e.g. InternalElements) in a context, e.g. a plant hierarchy to ensure that tools importing this model can locate the elements within their world. As this is a common methodology, AutomationML supports this by means of the Structure RoleClasses contained in the AutomationMLBaseRoleClassLib and derived RoleClasses, e.g. within the AutomationMLExtendedRoleClassLib. The AutomationMLExtendedRoleClassLib of [IEC62714-2 2015] defines specific RoleClasses which contain the semantics of hierarchical elements of [IEC61512 1997] and [IEC62264 2013] depicted in Fig. 1.7. All these ten RoleClasses have the RoleClass ResourceStructure of the AutomationMLBaseRoleClassLib as semantic parent.

Fig. 1.7: RoleClasses for modelling a hierarchical structure

Version 2.7.0 of the AutomationMLExtendedRoleClassLib (see Fig. 1.8) extends these RoleCasses by means of nine additional RoleClasses given in [IEC PAS 63088 2017] for the context of Industrie 4.0. This version of the AutomationMLExtendedRoleClassLib will be included in the upcoming second edition of [IEC62714-2 2015].

All 19 available RoleClasses to model resource structures are depicted in Fig. 1.9.

The relation between the present RoleClasses given by the underlying standards is depicted in Fig. 1.10. As defined in these standards, not all hierarchy levels are always mandatory. AutomationML defines no restrictions for the combination of these RoleClasses but references the underlying standards.

Fig. 1.8: AutomationMLExtendedRoleClassLib (Version 2.7.0)

Fig. 1.9: RoleClasses for modelling a hierarchical structure (AutomationMLExtendedRoleClassLib Version 2.7.0)

Fig. 1.10:Resource structure (SOURCE: [IEC PAS 63088 2017], adapted)

### Example

To create an example for a hierarchically structured plant, several steps must be done. First, an InstanceHierarchy containing the plant and all underlying elements (i.e. InternalElements) must be created. Second, the AutomationMLExtendedRoleClassLib must be imported into the model. Third, the corresponding semantics by means of RoleClasses must be assigned to the InternalElements. Additionally, SystemUnitClasses and Interfaces can be used/modelled as well as attributes and corresponding relations between all elements. This example only focusses the RoleClasses. The creation of the model will be described by means of the AutomationML Editor but can be done also directly in XML text.

The example shall contain two plants: Plant 1 and Plant 2. Plant 1 comprises of two production lines with three resp. two work cells. Plant 2 comprises of one production line with four work cells.

### Step 1: Create an InstanceHierarchy with InternalElements

Within the AutomationML Editor a new InstanceHiearchy can be created by simply clicking the plus symbol if there is not already one present in the model. This InstanceHierarchy shall be renamed to the specific context so that it can be distinguished easily in the case that two AutomationML files are merged. Another good practice is to immediately start with versioning the InstanceHierarchy and adjust the version number accordingly in case of changes. In the present example, the InstanceHiearchy is name RoleSampleInstanceHierarchy and its Version is 1.0.

Fig. 1.11: InstanceHierarchy creation in AutomationML Editor

Subsequent elements can be added by the same graphical symbol when selecting the InstanceHierarchy element. All elements can be nested accordingly to the intended or actual hierarchy. The InternalElements should be renamed to something meaningful or well-known in the context. A description can be added according to your own needs. The AutomationML editor automatically assigns unique identifiers to each of the InternalElements as the present standard [IEC62714-1 2018] recommends the identification of InternalElements and ExternalInterfaces by means of GUIDs. All references to these InternalElements are created by means of these IDs. The AutomationML Engine provides the same functionality in case of a programmatic creation of the model. In case of a manual creation, the editor must take care on this individually.

### Step 2: Apply the semantics (aka RoleClasses)

After the creation of the rough InstanceHiearchy, the AutomationMLExtendedRoleClassLib can be imported. This can be done as described in section 1.2, e.g.  by the menu entry “File”“Import””from Files” and the selection of the downloaded AutomationMLExtendedRoleClassLib. After the import the library appears in the corresponding visual section of the AutomationML editor for RoleClassLibs.

All relations to the corresponding roles can be created by a simple drag and drop of the RoleClasses from the AutomationMLExtendedRoleClassLib to the InternalElements within the InstanceHierarchy. The pre-selection in the displayed “Paste” dialog (see Fig. 1.12) can be kept as is.

Fig. 1.12: Paste dialog of the AutomationML Editor

The resulting InstanceHierarchy with all created InternalElements and assigned RoleClasses is depicted in Fig. 1.13 and can be found in the example AutomationML file [LINKAMLExample].

Fig. 1.13: Example InstanceHiarchy

## The ISA 106 hierarchy as example how to map a user-defined RoleClass library

User-defined RoleClass libraries can be defined by each user of AutomationML. One example which will be included in the upcoming second edition of [IEC62714-2 2015] is the mapping of hierarchical levels within ISA106 [ISATR106 2013]. These roles are not part of the AutomationMLExtendedRoleClassLibary but can be mapped to the corresponding roles as proposed in ISA106 (see Fig. 1.14). A RoleClass library for ISA 106 can be created as user-defined RoleClass library as depicted in Fig. 1.15.

| ISA 106 | ISA 88 |
| --- | --- |
| Enterprise | Enterprise |
| Site | Site |
| Plant | Area |
| PlantArea | ProcessCell |
| Unit | Unit |
| Equipment | EquipmentModule |
| Device | ControlModule |

Fig. 1.14: ISA 106 [ISATR106 2013] mapping to ISA88 [ISA88 1995]

Fig. 1.15: UserDefinedRoleClassLibISA106

## Discussion and conclusion

The full potential for an efficient information exchange across tool borders can only be opened if the exchanged information can be understood by all partners. This requires common semantics which are represented in RoleClasses in the context of AutomationML. A role describes an abstract functionality without defining the underlying technical implementation. Roles can help to map data models of different involved tools or partners.

A wide range of defined RolesClasses and their corresponding semantics is available in different quality: international standard, consortium standard, company standard, user-defined. Additionally, all AutomationML users can define new RoleClasses specific to their domain or use cases. Objective of the present chapter is to explain how to use existing and user-defined AutomationML RoleClasses and to mention some best practices in this field.

## Example files (online reference)

[LINKAMLExample]	www.dummyreference.de

## Literature

(IEC62714-1 2018)	IEC 62714-1:2018, Engineering data exchange format for use in industrial automation systems engineering - Automation Markup Language - Part 1: Architecture and general requirements, Edition 2.0, April 2018.

(IEC62714-2 2015)	IEC62714-2: 2015, Engineering data exchange format for use in industrial automation systems engineering - Automation markup language - Part 2: Role class libraries, Edition 1.0, March 2015.

(IEC62714-3 2017)	IEC 62714-3: 2017, Engineering data exchange format for use in industrial automation systems engineering - Automation markup language - Part 3: Geometry and kinematics, January 2017.

(IEC62714-4 2020)	IEC 62714-4:2020, Engineering data exchange format for use in industrial automation systems engineering - Automation markup language - Part 4: Logic, June 2020.

(IEC PAS 63088 2017)	IEC PAS 63088:2017, Smart manufacturing - Reference architecture model industry 4.0 (RAMI4.0), March 2017.

(AML1 2020)	AutomationML (2020) https://www.automationml.org/o.red.c/publications.html

(AML2 2020)	AutomationML standard libraries (2020) https://www.automationml.org/o.red/uploads/dateien/1485869403-AutomationML_Standard_Libraries_Jan2017.zip

(AML3 2020)	AutomationML libraries (2020), https://www.automationml.org/o.red/uploads/dateien/1494835189-AutomationML_Libraries_May2017.zip

(AML4 2020)	AutomationML Application Recommendation Modelling of Material Handling in AutomationML, April 2020, https://www.automationml.org/o.red/uploads/dateien/1596017115-AR_Modelling%20of%20Material%20Handling%20in%20AutomationML_v1_0_0.zip

(ISA88 1995)	ANSI/ISA-88.01-1995, Batch control – Part 1: Models and Terminology

(IEC61512 1997)	IEC 61512-1:1997,Batch control – Part 1: Models and terminology

(IEC62264 2013)	IEC 62264-1:2013, Enterprise-control system integration - Part 1: Models and terminology

(ISATR106 2013)	ISA-TR106.00.01-2013, Procedure Automation for Continuous Process Operations – Models and Terminology, August 2013

Stichwortverzeichnis

R

RoleClasses  1

S

semantics  1

## Inhaltsverzeichnis

1	Extended Libraries	1

1.1	How to use AutomationML RoleClass Libraries	1

1.1.1	User-defined RoleClass libraries in one model	1

1.1.2	User-defined RoleClass libraries in multiple models	2

1.1.3	User-defined RoleClass libraries in a defined context	2

1.1.4	AutomationMLBaseRoleClassLib	2

1.1.5	Normative AutomationML IEC RoleClass libraries	3

1.1.6	Informative AutomationML IEC RoleClass libraries	4

1.1.7	AutomationML e.V. RoleClass libraries	6

1.2	How to work with AutomationML RoleClass Libraries	6

1.3	How to model a “plant” hierarchy with the ResourceStructure semantic	8

1.3.1	Example	12

1.3.2	Step 1: Create an InstanceHierarchy with InternalElements	12

1.3.3	Step 2: Apply the semantics (aka RoleClasses)	13

1.4	The ISA 106 hierarchy as example how to map a user-defined RoleClass library	14

1.5	Discussion and conclusion	14

1.6	Example files (online reference)	15

1.7	Literature	15

1.8	Inhaltsverzeichnis	16